METHOD OF PROCEDURE Intelligent Campus Network Automation Design, Implementation & Management

| Module | EE8203 - Design and Management of Data Networks |
| --- | --- |
| Project | Group Project - Intelligent Campus Network Automation |
| Institution | Faculty of Engineering, University of Ruhuna |
| Department | Dept. of Computer Engineering (DCE) |
| Group Members | EG/2021/4520 Gangadara M.G.K EG/2021/4465 De Silva K.G.B.P |
| Date |  |

# 1. Document Objective

This Method of Procedure (MOP) documents the complete implementation plan for the EE8203 Group Project: a hierarchical campus data network for the Faculty of Engineering, University of Ruhuna, Hapugala. It covers all phases from initial design through to post-implementation review, following industry-standard MOP conventions.

This document is written in accordance with the EE8203 project specification (Section 9) and mirrors the structure of professional MOPs used in telecommunications and enterprise network operations. It is intended to guide two engineers through the full build, automation, monitoring, and verification of the faculty network in GNS3.

Change window The target implementation window is Week 9 of the module schedule. The total estimated duration is 8-10 hours for a full reset-and-replay. Individual phases can be executed independently during earlier weeks as per the project schedule.

# 2. Introduction

## 2.1 Scenario Overview

The Faculty of Engineering at the University of Ruhuna operates four academic and administrative departments. As the network engineering team, we are engaged to design, implement, automate, and monitor the campus data network interconnecting all four departments.

| Code | Department | Role in Network |
| --- | --- | --- |
| DEIE | Dept. of Electrical & Information Engineering | Engineering research & undergraduate labs - VLAN 10 |
| DCEE | Dept. of Civil & Environmental Engineering | Administration & project coordination - VLAN 20 |
| DMME | Dept. of Mechanical & Manufacturing Engineering | Workshop lab & server access zone - VLAN 30 |
| DIS | Dept. of Inter Disciplinary Studies | Faculty IT hub - hosts central server farm - VLAN 40 |

## 2.2 Network Architecture

The network follows a three-layer hierarchical design consistent with Cisco's recommended campus network model:

- Core layer - R-EDGE and R-CORE (Cisco 7200): WAN connectivity, NAT, OSPF backbone

- Distribution layer - SW-CORE (L3) + SW-D-DEIE, SW-D-DCEE, SW-D-DMME: inter-VLAN routing, ACL enforcement

- Access layer - SW-A-DEIE, SW-A-DCEE, SW-A-DMME, SW-A-DIS: end-device connectivity, VLAN assignment

- Management plane - VLAN 99 (10.99.99.0/24): out-of-band SSH and SNMP, OOB-SW, VM-AUTO, VM-ZABBIX, VM-DHCP

## 2.3 Device Inventory

| Hostname | Qty | GNS3 Image | Primary Role |
| --- | --- | --- | --- |
| R-EDGE | 1 | Cisco 7200 / IOSv | Edge router - default route, NAT overload (PAT), OSPF area 0 |
| R-CORE | 1 | Cisco 7200 / IOSv | Core router - OSPF area 0, inter-zone routing |
| SW-CORE | 1 | IOSvL2 / EtherSwitchRouter | Core L3 switch - inter-VLAN routing, SVI gateway |
| SW-D-DEIE | 1 | IOSvL2 | Distribution L3 switch - DEIE department |
| SW-D-DCEE | 1 | IOSvL2 | Distribution L3 switch - DCEE department |
| SW-D-DMME | 1 | IOSvL2 | Distribution L3 switch - DMME department |
| SW-A-DEIE | 1 | IOSvL2 | Access L2 switch - DEIE workstations |
| SW-A-DCEE | 1 | IOSvL2 | Access L2 switch - DCEE workstations |
| SW-A-DMME | 1 | IOSvL2 | Access L2 switch - DMME workstations |
| SW-A-DIS | 1 | IOSvL2 | Access L2 switch - DIS server farm (direct to SW-CORE) |
| VM-AUTO | 1 | Ubuntu 22.04 | Python/Netmiko automation + Ansible control node |
| VM-ZABBIX | 1 | Ubuntu 22.04 | Zabbix 6.x LTS - SNMPv2c monitoring server |
| VM-DHCP/WEB | 1 | Linux | DHCP server, DNS, web server for reachability tests |
| VPC end-hosts | 8+ | GNS3 VPC | Two per department - ACL policy testing |

# 3. Scope and Objectives

## 3.1 In Scope

- Full three-layer hierarchical campus network build in GNS3

- VLAN design and 802.1Q trunking across all switch layers

- Inter-VLAN routing via SW-CORE SVIs and OSPF area 0

- NAT overload (PAT) on R-EDGE - VLAN 10 (DEIE) and VLAN 20 (DCEE) only

- Extended named ACL implementation and verification (4×4 reachability matrix)

- Python/Netmiko automation for R-CORE and R-EDGE router configuration

- Ansible playbook automation for all distribution and access switches

- Zabbix 6.x monitoring: host onboarding, SNMPv2c, triggers, custom dashboard

- Out-of-band management network (VLAN 99, 10.99.99.0/24)

- Post-implementation review, rollback procedures, and all submission artefacts

## 3.2 Out of Scope

- Physical hardware procurement or cabling - GNS3 simulation only

- IPv6 addressing

- Wireless LAN (WLAN) design

- HSRP/redundancy configuration (optional enhancement, not assessed)

- Production deployment - this MOP covers the GNS3 simulation environment

## 3.3 Devices in Scope

All 13 configurable network devices listed in Section 2.3 are in scope. No devices have been removed from the minimum specification. An additional OOB-SW (management switch, VLAN 99) has been added to properly isolate the management plane - this is documented as an addition with justification in the design document.

# 4. Prerequisites and pre-checks

All prerequisite checks below must pass before proceeding to the implementation procedure. If any check fails, resolve the issue before continuing. Do not proceed with a failed pre-check.

| Item | Action | Expected result | Verification command |
| --- | --- | --- | --- |
| GNS3 environment | GNS3 application running and project file loaded | GNS3 status bar shows all nodes green | Visual check in GNS3 UI |
| All devices reachable | Ping sweep from VM-AUTO to all VLAN 99 addresses | All 10.99.99.x addresses respond | for ip in 10.99.99.{1..24}; do ping -c1 $ip; done |
| Ansible inventory | Ansible ping module against all hosts | All hosts return pong | ansible all -m ping -i inventory/hosts.yml |
| SSH access | SSH from VM-AUTO to each device on VLAN 99 | Login prompt received | ssh admin@10.99.99.10 |
| Zabbix web UI | HTTP GET to Zabbix login page | HTTP 200 response | curl -s -o /dev/null -w "%{http_code}" http://10.99.99.51/zabbix |
| GNS3 snapshot | Snapshot taken before each major phase | Snapshot appears in GNS3 project | GNS3 → Edit → Snapshots → Create |
| Git repository | Git repo initialised with at least Week 1 commit | git log shows history | git log --oneline |
| DHCP server | VM-DHCP/WEB service running | VPC receives IP via DHCP | VPC> dhcp (in GNS3 VPC terminal) |

Note Take a GNS3 snapshot named 'PRE-IMPLEMENTATION' immediately before Phase 1 begins. This is the master rollback point for the entire implementation.

# 5. Risk Assessment

| Risk | Likelihood | Impact | Mitigation |
| --- | --- | --- | --- |
| ACL blocks management SSH | Medium | High | Apply ACL only to data VLANs. Verify SSH from VLAN 99 before applying ACL. Test one interface before full deployment. |
| Automation script overwrites existing config | Low | Medium | Take GNS3 snapshot before each automation run. Test script on one device (VPC-D1) first. Use idempotency checks. |
| GNS3 node crashes mid-config | Medium | High | Save GNS3 project frequently. Keep configuration in Git. Restore from snapshot if node is unrecoverable. |
| OSPF adjacency not forming | Low | High | Verify interface IPs and subnet masks before enabling OSPF. Use 'debug ip ospf adj' if neighbour not appearing. |
| Zabbix SNMP polling fails | Low | Medium | Verify SNMPv2c community string matches on device and Zabbix host. Check ACL permits UDP 161 from 10.99.99.51. |
| Ansible playbook non-idempotent | Medium | Medium | Run 'ansible-playbook --check' before applying. Review task conditions. Use 'state: present' throughout. |
| Wrong VLAN on trunk port | Low | High | Verify 'show interfaces trunk' after each trunking step. Use 'switchport trunk allowed vlan' explicitly. |
| Git history incomplete at submission | Medium | High | Commit after every working phase from Week 1. Never leave more than 48 hours between commits. |

# 6. Step-by-Step Implementation Procedure

Implementation order Phases must be executed in order: 1 → 2 → 3 → 4 → 5 → 6 → 7. Do not skip a phase. Take a GNS3 snapshot after each phase completes successfully.

## Phase 1 - VLAN Design and Trunking

| Step | Action | Expected output | Verification command |
| --- | --- | --- | --- |
| 01 | Connect to SW-CORE via SSH from VM-AUTO | SSH session established | ssh admin@10.99.99.1 |
| 02 | Enter global config and create all VLANs | VLANs 10,20,30,40,99,100 created | vlan 10 name VLAN_DEIE |
| 03 | Verify VLANs on SW-CORE | All VLANs active in output | show vlan brief |
| 04 | Configure trunk uplinks SW-CORE → SW-D-* | Trunk ports showing VLANs allowed | show interfaces trunk |
| 05 | Repeat VLAN creation on each SW-D-* switch | VLANs present on distribution SWs | show vlan brief |
| 06 | Configure trunk: SW-D-DEIE → SW-A-DEIE | 802.1Q trunk active | show interfaces trunk |
| 07 | Configure access ports on SW-A-* switches | Access VLAN assigned to host ports | show interfaces status |
| 08 | Take GNS3 snapshot: 'PHASE1-VLANS-DONE' | Snapshot visible in GNS3 project | GNS3 UI → Snapshots |

## Phase 2 - Routing (OSPF, NAT, Default Route)

| Step | Action | Expected output | Verification command |
| --- | --- | --- | --- |
| 01 | Configure SVI on SW-CORE for VLAN 10 | Interface Vlan10 up/up | show ip interface brief |
| 02 | Configure SVIs for VLANs 20, 30, 40, 99 | All SVIs up/up | show ip interface brief |
| 03 | Configure routed port Gi0/0 on SW-CORE | 192.168.1.2/30 assigned | show ip interface brief |
| 04 | Configure R-CORE interfaces (Fa0/0, Fa0/1, Lo0) | All interfaces up | show ip interface brief |
| 05 | Configure R-EDGE interfaces (Fa0/0 WAN, Fa0/1, Lo0) | WAN link up | show ip interface brief |
| 06 | Enable OSPF area 0 on R-EDGE, R-CORE, SW-CORE | Neighbours forming | show ip ospf neighbor |
| 07 | Verify full routing table on R-CORE | All 10.10.x.0 subnets present | show ip route |
| 08 | Configure static default route on R-EDGE | 0.0.0.0/0 in routing table | show ip route |
| 09 | Configure NAT overload (PAT) on R-EDGE | NAT translations appearing | show ip nat translations |
| 10 | Take snapshot: 'PHASE2-ROUTING-DONE' | Snapshot saved | GNS3 UI → Snapshots |

## Phase 3 - Management Plane (VLAN 99, SSH, SNMP)

| Step | Action | Expected output | Verification command |
| --- | --- | --- | --- |
| 01 | Configure VLAN 99 SVI on SW-CORE (10.99.99.1/24) | SVI up/up | show ip interface brief |
| 02 | Configure VLAN 99 on all SW-D-* and SW-A-* switches | Mgmt IP reachable | ping 10.99.99.x |
| 03 | Enable SSH on all devices with domain and RSA key | SSH v2 enabled | show ip ssh |
| 04 | Configure 'enable secret' and 'username admin' | Auth working | ssh admin@10.99.99.10 |
| 05 | Verify SSH from VM-AUTO to all 10 devices | All logins succeed | for each device: ssh admin@10.99.99.x |
| 06 | Configure SNMPv2c community string on all devices | SNMP responding | snmpwalk -v2c -c public 10.99.99.10 |

## Phase 4 - ACL Implementation

| Step | Action | Expected output | Verification command |
| --- | --- | --- | --- |
| 01 | Design ACL on paper - map source/dest/action to interface/direction | ACL policy table complete | Review against spec Section 3.4 |
| 02 | Apply ACL_DEIE: permit VLAN 10 → VLAN 40 (any) | DEIE can reach DIS | ping 10.10.40.10 from VPC-D1 |
| 03 | Apply ACL_DCEE: permit HTTP/HTTPS to VLAN 40 only | DCEE reaches web on DIS only | curl http://10.10.40.52 from VPC-C1 |
| 04 | Apply ACL_DMME: deny all to VLAN 40 | DMME cannot reach DIS | ping 10.10.40.10 from VPC-M1 - timeout |
| 05 | Apply ACL: deny DCEE → DEIE | DCEE cannot reach DEIE | ping 10.10.10.10 from VPC-C1 - timeout |
| 06 | Apply ACL: deny DMME → DCEE | DMME cannot reach DCEE | ping 10.10.20.10 from VPC-M1 - timeout |
| 07 | Apply SSH ACL - permit 10.99.99.0/24 only | SSH only from VLAN 99 | ssh admin@10.99.99.10 from VLAN 10 - refused |
| 08 | Apply SNMP ACL - permit 10.99.99.51 (VM-ZABBIX) | SNMP polling works from Zabbix | snmpwalk from VM-ZABBIX succeeds |
| 09 | Populate 4×4 reachability matrix with screenshots | All cells verified | ping & traceroute from each VPC |
| 10 | Take snapshot: 'PHASE4-ACL-DONE' | Snapshot saved | GNS3 UI |

## Phase 5 - Netmiko Python Automation

| Step | Action | Expected output | Verification command |
| --- | --- | --- | --- |
| 01 | Install Python 3, Netmiko, paramiko on VM-AUTO | Libraries installed | pip3 show netmiko |
| 02 | Create device inventory file: inventory/devices.json | File readable and valid JSON | python3 -c "import json; json.load(open('devices.json'))" |
| 03 | Write netmiko_routers.py - interface IP + OSPF automation | Script exists and commented | cat netmiko_routers.py |
| 04 | Write netmiko_snmp.py - push SNMPv2c to all devices | Script pushes community string | cat netmiko_snmp.py |
| 05 | Wipe R-CORE config (write erase + reload) | R-CORE returns to factory default | show running-config - minimal |
| 06 | Run python3 netmiko_routers.py | R-CORE fully configured | show ip interface brief, show ip ospf neighbor |
| 07 | Run python3 netmiko_snmp.py | SNMP community on all devices | snmpwalk -v2c -c public 10.99.99.10 |
| 08 | Run scripts a second time - verify idempotency | No duplicate config entries | show run - no duplicate lines |
| 09 | Review timestamped log file | Log file created with output | cat logs/netmiko_$(date +%Y%m%d)*.log |

## Phase 6 - Ansible Playbook Automation

| Step | Action | Expected output | Verification command |
| --- | --- | --- | --- |
| 01 | Install Ansible and cisco.ios collection on VM-AUTO | Collection installed | ansible-galaxy collection list ¦ grep cisco.ios |
| 02 | Create role structure: vlans, trunking, access_ports, stp | Directories created | ls roles/ |
| 03 | Populate group_vars/all.yml and host_vars/sw-d-deie.yml etc. | Variables accessible | ansible-playbook site.yml --list-vars |
| 04 | Write and run vlans role against SW-D-DEIE first | VLANs created on test switch | ansible-playbook site.yml --limit sw-d-deie --tags vlans |
| 05 | Run site.yml against all switches | All switches configured | ansible-playbook site.yml |
| 06 | Run idempotency check | Zero changes reported | ansible-playbook site.yml --check |
| 07 | Commit all playbooks to Git with meaningful messages | Git log shows phase history | git log --oneline |
| 08 | Test rollback.yml on one access switch | Switch returns to baseline | ansible-playbook playbooks/rollback/rollback.yml --limit sw-a-dcee |

## Phase 7 - Zabbix Monitoring

| Step | Action | Expected output | Verification command |
| --- | --- | --- | --- |
| 01 | Install Zabbix 6.x LTS on VM-ZABBIX (Ubuntu 22.04) | Zabbix web UI accessible | curl http://10.99.99.51/zabbix → HTTP 200 |
| 02 | Add all 10 network devices as Zabbix hosts | Hosts appear in Monitoring → Hosts | Zabbix UI — host list |
| 03 | Apply 'Template Net Cisco IOS SNMPv2' to all Cisco devices | Template linked | Zabbix UI → Host → Templates |
| 04 | Configure trigger: device unreachable (3 polls) | Trigger appears in configuration | Zabbix UI → Trigger list |
| 05 | Configure trigger: interface down | Trigger fires within one poll | Shut GNS3 interface, wait for alert |
| 06 | Configure trigger: CPU > 80% for 60 s | Trigger active | Zabbix UI → Trigger list |
| 07 | Add one custom trigger (student-defined) | Custom trigger documented and active | Zabbix UI |
| 08 | Create dashboard named 'FoE-UoR Network' | Dashboard visible and complete | Zabbix UI → Dashboards |
| 09 | Export dashboard as JSON | JSON file saved | Zabbix → Configuration → Export |
| 10 | Demonstrate trigger fire: shut an interface in GNS3 | Alert appears in Zabbix within polling interval | Zabbix → Problems view |

# 7. Test Plan

## 7.1 4×4 Department Reachability Matrix

Each cell must be verified by ping and traceroute from the relevant source VPC. Screenshot evidence is required for every cell. ✓ = permitted, ✗ = denied as expected.

| Source \ Destination | DEIE (10.10.10.x) | DCEE (10.10.20.x) | DMME (10.10.30.x) | DIS (10.10.40.x) |
| --- | --- | --- | --- | --- |
| DEIE (10.10.10.x) | - | DENY (no policy) | DENY (no policy) | PERMIT (full) |
| DCEE (10.10.20.x) | DENY (ACL) | - | DENY (ACL) | PERMIT HTTP/S only |
| DMME (10.10.30.x) | DENY (no policy) | DENY (ACL) | - | DENY (ACL) |
| DIS (10.10.40.x) | PERMIT (full) | PERMIT HTTP/S only | DENY (ACL) | - |

## 7.2 Automation Verification

| Test | Method | Pass criteria |
| --- | --- | --- |
| Netmiko reset-and-replay | Wipe R-CORE config, run netmiko_routers.py | R-CORE fully configured within 5 minutes |
| Netmiko idempotency | Run script twice - compare 'show run' | No duplicate config lines on second run |
| Ansible idempotency | Run ansible-playbook site.yml --check after application | Zero changed tasks reported |
| Ansible rollback | Run rollback.yml on one access switch | Switch returns to clean baseline |
| Git history review | git log --oneline | Commits span all 9 weeks with meaningful messages |

## 7.3 Zabbix Monitoring Verification

| Test | Method | Pass criteria |
| --- | --- | --- |
| Device unreachable trigger | Suspend a GNS3 node | Alert fires within 3 poll intervals |
| Interface down trigger | Shut interface in GNS3 config | Alert fires within one poll cycle |
| CPU trigger | N/A - demonstrated via manual trigger test | Trigger visible in Zabbix Problems |
| Custom trigger (student-defined) | Per student definition | Trigger fires as configured |
| Dashboard completeness | Open 'FoE-UoR Network' dashboard | Host map, traffic graphs, trigger count visible |
| Dashboard JSON export | Zabbix → Configuration → Export | JSON file downloadable |

# 8. Implementation Time Plan

| Week | Phase | Activities | Deliverable | Owner |
| --- | --- | --- | --- | --- |
| Week 1 | Design phase | IP plan, VLAN scheme, topology diagram, device justification, GNS3 project skeleton | Design doc + topology diagram (PDF) | Both |
| Weeks 2–3 | Baseline config | Manual VLAN, trunking, OSPF, NAT, VLAN 99 SSH config. Full reachability verified. | GNS3 snapshot + connectivity log | Both |
| Week 4 | ACL implementation | Design and apply all inter-department ACLs. 4×4 matrix populated. | ACL table + test screenshots | Both |
| Weeks 5–6 | Netmiko (Python) | Write and test Python scripts. Code review in Week 6 lab. | Python scripts + execution log | Engineer 1 |
| Weeks 7–8 | Ansible (switches) | Write playbooks with roles. Idempotency tested. Git history complete. | Ansible Git repo + playbook output | Engineer 2 |
| Week 9 | Zabbix monitoring | Deploy Zabbix. Onboard all devices. Configure triggers and dashboard. | Dashboard screenshot + trigger evidence | Both |
| Week 9 | MOP + submission | Complete MOP. Full reset-and-replay test. Record 10-min demo video. | All submission artefacts | Both |

# 9. Rollback Procedure

A rollback procedure is documented for each major phase. The primary rollback mechanism is GNS3 snapshots. Snapshots must be taken before each phase begins. The target rollback time for any single device is under 5 minutes.

Master rollback point GNS3 snapshot 'PRE-IMPLEMENTATION' is the master rollback point for the entire project. Restoring this snapshot returns all devices to the verified baseline state from the end of Week 3.

### Rollback 1: ACL blocks SSH management access

Connect via GNS3 console (not SSH) to affected device. Remove ACL from interface: no ip access-group [ACL_NAME] in. Re-apply with corrected permit for 10.99.99.0/24 first.

### Rollback 2: Netmiko script corrupts router config

Stop script. Restore from GNS3 snapshot taken before script run. Snapshot name: 'PRE-NETMIKO-[device]'. Reload node from snapshot in GNS3 UI.

### Rollback 3: Ansible playbook causes VLAN mismatch

Run rollback.yml playbook: ansible-playbook playbooks/rollback/rollback.yml --limit [switch]. Verify with show vlan brief. Restore from snapshot if playbook fails.

### Rollback 4: OSPF adjacency lost after change

Issue 'no router ospf 1' and re-apply OSPF config. Verify with show ip ospf neighbor. If unrecoverable, restore R-CORE from snapshot.

### Rollback 5: Wrong VLAN assigned to access port

Re-enter switch config: interface [Fa/Gi], switchport access vlan [correct VLAN]. Verify: show interfaces status.

### Rollback 6: Full implementation failure

Restore entire GNS3 project from snapshot 'PRE-IMPLEMENTATION'. All devices return to pre-change state. Estimated time: < 5 minutes.

# 10. Persons in Charge (PIC)

| Role | Name | Student ID | Responsibility |
| --- | --- | --- | --- |
| Engineer 1 (Lead) | Gangadara M.G.K. | EG/2021/4520 | Routing, Netmiko, Zabbix |
| Engineer 2 | De Silva K.G.B.P | EG/2021/4465 | Switching, Ansible, ACLs |
| Academic Supervisor |  | - | Assessment and guidance |

# 11. Post-Implementation Review

Note This section is completed after the project is finished, before final submission. It contributes to the MOP quality mark. Be honest — documenting what went wrong demonstrates professional maturity.

## 11.1 What Worked as Planned

## 11.2 Deviations from the MOP

| Phase | What changed | Reason and resolution |
| --- | --- | --- |

## 11.3 Lessons Learned

## 11.4 Submission Checklist

| ✓ | Artefact | Filename |
| --- | --- | --- |
| ☐ | Python scripts (Netmiko) | EE8203_[ID1_ID2]_Netmiko.zip |
| ☐ | Ansible Git repository link | GitHub/GitLab URL in report |
| ☐ | Zabbix dashboard JSON export | EE8203_[ID1_ID2]_Zabbix.json |
| ☐ | MOP document (PDF + .docx) | EE8203_[ID1_ID2]_MOP.pdf / .docx |
| ☐ | Final technical report (PDF, max 20 pages) | EE8203_[ID1_ID2]_Report.pdf |
| ☐ | 10-minute demonstration video (MP4) | EE8203_[ID1_ID2]_Demo.mp4 |
| ☐ | 4×4 reachability matrix with screenshots | Included in report appendix |

# Appendix A - IP Address Plan

## A.1 VLAN Scheme

| VLAN | Name | Dept | Network | Mask | Pfx | Gateway | Purpose |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | VLAN_DEIE | DEIE | 10.10.10.0 | 255.255.255.0 | /24 | 10.10.10.1 | Engineering workstations |
| 20 | VLAN_DCEE | DCEE | 10.10.20.0 | 255.255.255.0 | /24 | 10.10.20.1 | Admin/civil dept hosts |
| 30 | VLAN_DMME | DMME | 10.10.30.0 | 255.255.255.0 | /24 | 10.10.30.1 | Workshop lab hosts |
| 40 | VLAN_DIS | DIS | 10.10.40.0 | 255.255.255.0 | /24 | 10.10.40.1 | Faculty server farm |
| 99 | MGMT | All | 10.99.99.0 | 255.255.255.0 | /24 | 10.99.99.1 | OOB management SSH/SNMP |
| 100 | NATIVE | Trunk | - | - | - | - | Native VLAN - no IP |

## A.2 Device Interface Addresses

| Device | Interface | IP address | Pfx | Description |
| --- | --- | --- | --- | --- |
| R-EDGE | Fa0/0 (WAN) | 203.0.113.1 | /30 | ISP WAN link |
| R-EDGE | Fa0/1 | 192.168.0.1 | /30 | Link to R-CORE |
| R-EDGE | Loopback0 | 1.1.1.1 | /32 | OSPF Router ID |
| R-CORE | Fa0/0 | 192.168.0.2 | /30 | Link to R-EDGE |
| R-CORE | Fa0/1 | 192.168.1.1 | /30 | Link to SW-CORE |
| R-CORE | Loopback0 | 2.2.2.2 | /32 | OSPF Router ID |
| SW-CORE | Gi0/0 (routed) | 192.168.1.2 | /30 | Uplink to R-CORE |
| SW-CORE | Vlan10 SVI | 10.10.10.1 | /24 | Gateway — DEIE |
| SW-CORE | Vlan20 SVI | 10.10.20.1 | /24 | Gateway — DCEE |
| SW-CORE | Vlan30 SVI | 10.10.30.1 | /24 | Gateway — DMME |
| SW-CORE | Vlan40 SVI | 10.10.40.1 | /24 | Gateway — DIS |
| SW-CORE | Vlan99 SVI | 10.99.99.1 | /24 | Gateway — MGMT |
| SW-D-DEIE | Vlan99 | 10.99.99.10 | /24 | Management interface |
| SW-D-DCEE | Vlan99 | 10.99.99.11 | /24 | Management interface |
| SW-D-DMME | Vlan99 | 10.99.99.12 | /24 | Management interface |
| SW-A-DEIE | Vlan99 | 10.99.99.20 | /24 | Management interface |
| SW-A-DCEE | Vlan99 | 10.99.99.21 | /24 | Management interface |
| SW-A-DMME | Vlan99 | 10.99.99.22 | /24 | Management interface |
| SW-A-DIS | Vlan99 | 10.99.99.23 | /24 | Management interface |
| VM-AUTO | eth0 | 10.99.99.50 | /24 | Automation host |
| VM-ZABBIX | eth0 | 10.99.99.51 | /24 | Monitoring server |
| VM-DHCP/WEB | eth0 | 10.99.99.52 | /24 | DHCP / DNS / web |
| VPC-D1 | VLAN 10 | 10.10.10.10 | /24 | DEIE endpoint 1 |
| VPC-D2 | VLAN 10 | 10.10.10.11 | /24 | DEIE endpoint 2 |
| VPC-C1 | VLAN 20 | 10.10.20.10 | /24 | DCEE endpoint 1 |
| VPC-C2 | VLAN 20 | 10.10.20.11 | /24 | DCEE endpoint 2 |
| VPC-M1 | VLAN 30 | 10.10.30.10 | /24 | DMME endpoint 1 |
| VPC-M2 | VLAN 30 | 10.10.30.11 | /24 | DMME endpoint 2 |
| VPC-S1 | VLAN 40 | 10.10.40.20 | /24 | DIS VPC |
| Server-1 | VLAN 40 | 10.10.40.10 | /24 | DIS server 1 |
| Server-2 | VLAN 40 | 10.10.40.11 | /24 | DIS server 2 |